Draperies and Stage Tracks General Specification.      Ash-Stevenson Inc.				2004

21 Progress Ave Units 10 & 11, Scarborough, Ontario, Canada M1P 4S8  (416) 321-1412  Fax: (416) 321-1086

Lighting / Support grid

OWJ

GRID

WALL

FLANGE

Bill of Material  - LAYOUT # 6

For support of  Lighting,  Draperies & tracks

Lighting / Support grid

(a)	Shall be an over/under design,  modular construction manufactured from 1 1/2” schedule 40 I.D. Structual Grade aluminum pipe supported at a minimum 6 foot intervals.  Grid shall be  a 4 x 4 foot or  5 x 5 foot design.  Provisional spacing of side pipes shall allow for the support of angled leg pipes and  or lifting mechanism. An 8:1 design factor for safety shall be maintained.

(b)	Grid suspension shall be via  3/8” threaded rod to OWJ and shall be stabilized from movement with wall flanges every second pipe anchored to the walls.

(c)	Grid cross-over fittings shall be used at pipe intersections. Internal pipe splices shall be used as required. Pipe fittings shall be Alvin or equivalent.

(d)	Elevation of grid shall be typically 14-16 ft. and shall be level.  Co-ordination of mechanical & electrical trades shall be by the general contractor.   Electrical contractor to ensure that general lighting fixtures are above the grid and any attachment distribution boxes are co-ordinated with lighting fixtures.

Over-under pipe clamps are an acceptable alternative.  In-line pipe fittings shall not be used unless support points are provided on each sectional pipe.